Appendix 2.

Motorola 68000 Instruction Set.

Condition Codes

---------------

Assembler   Data

Instruction Description                    Syntax     Size        X N Z V C

-----------------------                   ---------   ----        ---------

ABCD     Add BCD with extend                Dx,Dy      B--        * U * U *

-(Ax),-(Ay)

ADD      ADD binary                        Dn,<ea>     BWL        * * * * *

<ea>,Dn

ADDA     ADD binary to An                  <ea>,An     -WL        - - - - -

ADDI     ADD Immediate                     #x,<ea>     BWL        * * * * *

ADDQ     ADD 3-bit immediate             #<1-8>,<ea>   BWL        * * * * *

ADDX     ADD eXtended                       Dy,Dx      BWL        * * * * *

-(Ay),-(Ax)

AND      Bit-wise AND                      <ea>,Dn     BWL        - * * 0 0

Dn,<ea>

ANDI     Bit-wise AND with Immediate    #<data>,<ea>   BWL        - * * 0 0

ASL      Arithmetic Shift Left            #<1-8>,Dy    BWL        * * * * *

Dx,Dy

<ea>

ASR      Arithmetic Shift Right              ...       BWL        * * * * *

Bcc      Conditional Branch            Bcc.S <label>   BW-        - - - - -

Bcc.W <label>

BCHG     Test a Bit and CHanGe             Dn,<ea>     B-L        - - * - -

#<data>,<ea>

BCLR     Test a Bit and CLeaR                ...       B-L        - - * - -

BSET     Test a Bit and SET                  ...       B-L        - - * - -

BSR      Branch to SubRoutine          BSR.S <label>   BW-        - - - - -

BSR.W <label>

BTST     Bit TeST                          Dn,<ea>     B-L        - - * - -

#<data>,<ea>

CHK      CHecK Dn Against Bounds           <ea>,Dn     -W-        - * U U U

CLR      CLeaR                              <ea>       BWL        - 0 1 0 0

CMP      CoMPare                           <ea>,Dn     BWL        - * * * *

CMPA     CoMPare Address                   <ea>,An     -WL        - * * * *

CMPI     CoMPare Immediate              #<data>,<ea>   BWL        - * * * *

CMPM     CoMPare Memory                  (Ay)+,(Ax)+   BWL        - * * * *

DBcc     Looping Instruction          DBcc Dn,<label>  -W-        - - - - -

DIVS     DIVide Signed                     <ea>,Dn     -W-        - * * * 0

DIVU     DIVide Unsigned                   <ea>,Dn     -W-        - * * * 0

EOR      Exclusive OR                      Dn,<ea>     BWL        - * * 0 0

EORI     Exclusive OR Immediate         #<data>,<ea>   BWL        - * * 0 0

EXG      Exchange any two registers         Rx,Ry      --L        - - - - -

EXT      Sign EXTend                         Dn        -WL        - * * 0 0

ILLEGAL  ILLEGAL-Instruction Exception     ILLEGAL                - - - - -

JMP      JuMP to Affective Address          <ea>                  - - - - -

JSR      Jump to SubRoutine                 <ea>                  - - - - -

LEA      Load Effective Address            <ea>,An     --L        - - - - -

LINK     Allocate Stack Frame       An,#<displacement>            - - - - -

LSL      Logical Shift Left                 Dx,Dy      BWL        * * * 0 *

#<1-8>,Dy

<ea>

Condition Codes (continued)

---------------

Assembler   Data

Instruction Description                    Syntax     Size        X N Z V C

-----------------------                   ---------   ----        ---------

LSR      Logical Shift Right                 ...       BWL        * * * 0 *

MOVE     Between Effective Addresses      <ea>,<ea>    BWL        - * * 0 0

MOVE     To CCR                           <ea>,CCR     -W-        I I I I I

MOVE     To SR                             <ea>,SR     -W-        I I I I I

MOVE     From SR                           SR,<ea>     -W-        - - - - -

MOVE     USP to/from Address Register      USP,An      --L        - - - - -

An,USP

MOVEA    MOVE Address                      <ea>,An     -WL        - - - - -

MOVEM    MOVE Multiple            <register list>,<ea> -WL        - - - - -

<ea>,<register list>

MOVEP    MOVE Peripheral                  Dn,x(An)     -WL        - - - - -

x(An),Dn

MOVEQ    MOVE 8-bit immediate         #<-128.+127>,Dn  --L        - * * 0 0

MULS     MULtiply Signed                   <ea>,Dn     -W-        - * * 0 0

MULU     MULtiply Unsigned                 <ea>,Dn     -W-        - * * 0 0

NBCD     Negate BCD                         <ea>       B--        * U * U *

NEG      NEGate                             <ea>       BWL        * * * * *

NEGX     NEGate with eXtend                 <ea>       BWL        * * * * *

NOP      No OPeration                        NOP                  - - - - -

NOT      Form one's complement              <ea>       BWL        - * * 0 0

OR       Bit-wise OR                       <ea>,Dn     BWL        - * * 0 0

Dn,<ea>

ORI      Bit-wise OR with Immediate     #<data>,<ea>   BWL        - * * 0 0

PEA      Push Effective Address             <ea>       --L        - - - - -

RESET    RESET all external devices         RESET                 - - - - -

ROL      ROtate Left                      #<1-8>,Dy    BWL        - * * 0 *

Dx,Dy

<ea>

ROR      ROtate Right                        ...       BWL        - * * 0 *

ROXL     ROtate Left with eXtend             ...       BWL        * * * 0 *

ROXR     ROtate Right with eXtend            ...       BWL        * * * 0 *

RTE      ReTurn from Exception               RTE                  I I I I I

RTR      ReTurn and Restore                  RTR                  I I I I I

RTS      ReTurn from Subroutine              RTS                  - - - - -

SBCD     Subtract BCD with eXtend           Dx,Dy      B--        * U * U *

-(Ax),-(Ay)

Scc      Set to -1 if True, 0 if False      <ea>       B--        - - - - -

STOP     Enable & wait for interrupts      #<data>                I I I I I

SUB      SUBtract binary                   Dn,<ea>     BWL        * * * * *

<ea>,Dn

SUBA     SUBtract binary from An           <ea>,An     -WL        - - - - -

SUBI     SUBtract Immediate                #x,<ea>     BWL        * * * * *

SUBQ     SUBtract 3-bit immediate       #<data>,<ea>   BWL        * * * * *

SUBX     SUBtract eXtended                  Dy,Dx      BWL        * * * * *

-(Ay),-(Ax)

SWAP     SWAP words of Dn                    Dn        -W-        - * * 0 0

TAS      Test & Set MSB & Set N/Z-bits      <ea>       B--        - * * 0 0

TRAP     Execute TRAP Exception           #<vector>               - - - - -

TRAPV    TRAPV Exception if V-bit Set       TRAPV                 - - - - -

TST      TeST for negative or zero          <ea>       BWL        - * * 0 0

UNLK     Deallocate Stack Frame              An                   - - - - -

Symbol   Meaning

------   -------

*     Set according to result of operation

-     Not affected

0     Cleared

1     Set

U     Outcome (state after operation) undefined

I     Set by immediate data

<ea>     Effective Address Operand

<data>   Immediate data

<label>  Assembler label

<vector> TRAP instruction Exception vector (0-15)

<rg.lst> MOVEM instruction register specification list

<displ.> LINK instruction negative displacement

...      Same as previous instruction

Addressing Modes                                   Syntax

----------------                                   ------

Data Register Direct                                 Dn

Address Register Direct                              An

Address Register Indirect                           (An)

Address Register Indirect with Post-Increment       (An)+

Address Register Indirect with Pre-Decrement        -(An)

Address Register Indirect with Displacement         w(An)

Address Register Indirect with Index               b(An,Rx)

Absolute Short                                        w

Absolute Long                                         l

Program Counter with Displacement                   w(PC)

Program Counter with Index                         b(PC,Rx)

Immediate                                            #x

Status Register                                      SR

Condition Code Register                              CCR

Legend

------

Dn    Data Register        (n is 0-7)

An    Address Register     (n is 0-7)

b    08-bit constant

w    16-bit constant

l    32-bit constant

x    8-, 16-, 32-bit constant

Rx    Index Register Specification, one of:

Dn.W  Low 16 bits of Data Register

Dn.L  All 32 bits of Data Register

An.W  Low 16 bits of Address Register

An.L  All 32 bits of Address Register

Condition Codes for Bcc, DBcc and Scc Instructions.

---------------------------------------------------

Condition Codes set after CMP D0,D1 Instruction.

Relationship      Unsigned                         Signed

------------      --------                         ------

D1 <  D0          CS - Carry Bit Set               LT - Less Than

D1 <= D0          LS - Lower or Same               LE - Less than or Equal

D1  = D0          EQ - Equal (Z-bit Set)           EQ - Equal (Z-bit Set)

D1 != D0          NE - Not Equal (Z-bit Clear)     NE - Not Equal (Z-bit Clear)

D1 >  D0          HI - HIgher than                 GT - Greater Than

D1 >= D0          CC - Carry Bit Clear             GE - Greater than or Equal

PL - PLus (N-bit Clear)          MI - Minus (N-bit Set)

VC - V-bit Clear (No Overflow)   VS - V-bit Set (Overflow)

RA - BRanch Always

DBcc Only    -     F - Never Terminate (DBRA is an alternate to DBF)

T - Always Terminate

Scc Only     -    SF - Never Set

ST - Always Set