US300

Two Hours

UNIVERSITY OF MANCHESTER

INSTITUTE OF SCIENCE AND TECHNOLOGY

CT310    Image Processing

For candidates taking:

DEGREE OF BSc IN COMPUTATION

FINAL EXAMINATION

DEGREE OF BSc IN INFORMATION ENGINEERING

FINAL EXAMINATION

DEGREE OF BSc IN COMPUTATION AND FRENCH

FINAL EXAMINATION

DEGREE OF BSc IN COMPUTATION AND GERMAN

FINAL EXAMINATION

DEGREE OF BSc IN PURE MATHEMATICS AND COMPUTATION

SECOND EXAMINATION

DEGREE OF BEng, MEng IN SOFTWARE ENGINEERING

THIRD YEAR SESSIONAL

DEGREE OF BEng, MEng IN COMPUTER SYSTEMS ENGINEERING

THIRD YEAR SESSIONAL

DEGREE OF BEng, MEng IN MICROELECTRONIC SYSTEMS ENGINEERING

THIRD YEAR SESSIONAL

DEGREE OF BEng, MEng IN ELECTRONIC SYSTEMS ENGINEERING

THIRD YEAR SESSIONAL

Thursday 22 January 1998								9:30-11:30

Answer three questions

Note: Do not answer more than the required number of questions.    Clearly cross out anything you do not wish to be marked

PTO

(2)

1)	Discuss the advantages and disadvantages of

(i) 	lossless and lossy image coding schemes

(5 marks)

(ii) 	fixed length and variable length codes

(5 marks)

Show, using the CCITT Group3 bilevel coding scheme as an example, how several coding algorithms can be applied successively to an image in order to achieve the required amount of compresion.

(10 marks)

2)	Explain, giving examples, what is meant by the spatial domain and frequency domain in the context of a digital image, and describe how these domains are related.

(10 marks)

Describe how the restoration of sinusoidal periodic degradations can be performed in the spatial and the frequency domain.

(10 marks)

3)	What is the goal of segmenting an image?

(5 marks)

One or more images in a longer sequence is believed to contain a region corresponding to an object that is of a different brightness to the remainder of the image. What method would you use to separate the region from the background in images that contain the region (12 marks), and how would you decide which images did not contain the region (3 marks)?

US300

(3)

4)	How is an edge defined?

(4 marks)

Discuss ways in which an image containing an object whose local statistics differ from its surroundings might be preprocessed to give grey-scale edges.

(6 marks)

Hence, describe the process of deriving the contour that surrounds this object in the image.

(10 marks)

5)	A set of objects are believed to be of a different colour to their background. How might the objects be separated from the background and the individual objects separated?

(12 marks)

In a sequence of images taken as the objects move in front of the camera, how might the motion of each object be inferred from their instantaneous locations determined in each image of the sequence?

(8 marks)

END OF PAPER